Design & Development Matrix

4-Layer Environmental Sensor PCB - Arduino Nano ESP32

Student ID: ab4647 | Document ID: QMS-FS-001 | Version: 1.1 | Date: March 2026

| Category | User Need | Target Specification | Alternative Solutions | Chosen Solution | Selection Criteria & Weighting | Verification Method | Verification Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Performance | System must regulate a stable 3.3 V supply from a battery input (6-9 V) with minimal ripple to power ESP32 and I2C sensors reliably. | Output voltage: 3.3 V +/-2% Ripple: <50 mV pk-pk Input range: 4.5-9 V Efficiency: >85% at 100 mA [1] | 1) RECOM R-6212P buck converter 2) Texas Instruments TPS62203 buck 3) AMS1117-3.3 LDO linear regulator | RECOM R-6212P buck converter | See Choice Matrix CM-01 | TP-VR-001: Measure VOUT with DMM under load; measure ripple with oscilloscope. | PASS - See TP-VR-001 results document. |
| Performance | Sensor must measure temperature and relative humidity with sufficient accuracy for long-term environmental monitoring. | Temperature accuracy: +/-0.3 C RH accuracy: +/-2% Interface: I2C Supply: 3.3 V compatible [2] | 1) Sensirion SHT31 (temp + RH) 2) Bosch BME680 (temp + RH + gas + pressure) 3) DHT22 (temp + RH, single-wire) | Sensirion SHT31 as primary; BME680 as secondary for gas/pressure | See Choice Matrix CM-02 | Visual inspection of I2C address response during bring-up. | Deferred - board not yet fabricated. |
| Performance | PCB must maintain signal integrity on I2C bus to ensure reliable communication between ESP32 and sensors. | SDA/SCL routed over continuous ground plane Separation from switching noise >5 mm Optional series damping resistors available [3] | 1) Route I2C on top layer over GND plane with series resistor option 2) Route I2C on inner layer with no damping 3) Use shielded flex cable off-board | Option 1: Top-layer I2C over GND plane with 0 ohm placeholders for optional damping | See Choice Matrix CM-03 | TP-SI-001: Probe SDA/SCL with oscilloscope; check rise time, overshoot, and noise margin. | PASS - See TP-SI-001 results document. |
| Societal | The sensor board should support safe, non-invasive environmental monitoring that does not pose health risks to users or communities near deployment sites. | RoHS compliant components No hazardous emissions during operation Low radiated EMI (CE pre-compliance) Data collected locally (no mandatory cloud dependency) [4] | 1) Fully RoHS-compliant BOM with local data storage 2) Non-RoHS components with cloud-only data pipeline 3) Partially compliant BOM with external antenna (higher EMI risk) | Option 1: Fully RoHS-compliant BOM with local data storage on ESP32 | RoHS compliance eliminates hazardous substances from the device. Local storage avoids societal concerns around data privacy and cloud dependency. Low EMI is supported by the 4-layer stack-up with internal GND plane. [4][5] | BOM review against RoHS substance list; EMI pre-compliance check via layout review. | PASS - All components RoHS compliant per supplier declarations. Layout reviewed for EMI. |
| Sustainability | PCB laminate material should minimise environmental harm while remaining manufacturable and cost-effective. | RoHS compliant Halogen-free preferred Recyclable via WEEE channels FR-4 baseline with mitigation plan [4][6] | 1) Standard FR-4 (TG 130-140) 2) Halogen-free FR-4 equivalent 3) Rogers high-frequency laminate | Standard FR-4 with halogen-free alternative noted for future revision | FR-4 chosen for cost, availability, and JLCPCB compatibility. Halogen-free flagged as preferred if supplier supports it at similar cost. [6][7] | Material declaration review against RoHS directive. | PASS - FR-4 is RoHS compliant. Halogen-free option identified for future. |
| Usability | Board must include test points and debug access to support development, fault-finding, and reuse. | Test points on: GND, VBAT, 3V3, SDA, SCL, EN, FB, VBAT_RAW Minimum pad size: 1.0 mm Accessible with standard probe [8] | 1) Dedicated test-point pads on top layer 2) Exposed vias only 3) Header pins for probing | Dedicated test-point pads on top layer | Test pads are lowest profile, easiest to probe, and do not add connector cost. Header pins add height and BOM cost. Exposed vias risk solder wicking and are harder to probe reliably. [8] | Visual inspection of layout for pad size and accessibility. | PASS - All test points present and accessible. |
| Aesthetic | Board layout should be clean, logically zoned, and suitable for portfolio presentation. | Functional zones clearly separated (power, digital, sensor) Silkscreen labels on all connectors and test points No unrouted nets or DRC errors | 1) Manual placement with logical zoning 2) Auto-place with manual adjustment 3) Compact random placement | Manual placement with logical zoning | Manual zoning gives best control over thermal separation and signal routing. Auto-place does not respect thermal or SI constraints. [9] | DRC pass in KiCad; visual review of layout. | PASS - Zero DRC errors; layout reviewed. |
| Business | PCB must be affordable for a student prototype run (qty 5) using a standard low-cost manufacturer. | Total board cost (qty 5): <$15 USD Manufacturer: JLCPCB or equivalent Standard lead time: 5-7 days [10] | 1) 2-layer PCB ($2.00 per 5, JLCPCB) 2) 4-layer PCB ($7.00 per 5, JLCPCB) 3) 6-layer PCB (~$25+ per 5) | 4-layer PCB at $7.00 (qty 5) + $23.58 shipping = $30.58 total | See Choice Matrix CM-04 | Quote obtained from JLCPCB online calculator. | PASS - Quote confirmed: $30.58 total. |

References

[1] RECOM Power GmbH, "R-6xxxP(D) Series: Switching Regulator (DC/DC Converter) - Datasheet," rev. unknown, pp. 1-2. Accessed: 28-Jan-2026.

[2] Sensirion AG, "SHT3x-DIS Datasheet: Humidity and Temperature Sensor," version 6, May 2019.

[3] NXP Semiconductors, "UM10204: I2C-bus specification and user manual," Rev. 7.0, Oct. 2021.

[4] European Union, "Directive 2011/65/EU of the European Parliament (RoHS)," EUR-Lex. Accessed: Feb. 3, 2026.

[5] European Commission, "General Data Protection Regulation (GDPR) - Regulation (EU) 2016/679," EUR-Lex. Accessed: Mar. 2026.

[6] A. C. Marques, A. S. O. Santos, and S. M. F. Correia, "A review of the recycling of non-metallic fractions of printed circuit boards," SpringerPlus, 2013.

[7] U.S. Environmental Protection Agency, "Flame Retardants in Printed Circuit Boards," U.S. EPA (fact sheet), 2015.

[8] IPC, "IPC-7251: Generic Requirements for Through-Hole Design and Land Pattern Standard," 2005.

[9] H. W. Ott, Electromagnetic Compatibility Engineering, Wiley, 2009.

[10] JLCPCB, "PCB Manufacturing Capabilities and Pricing," online calculator. Accessed: Jan. 2026.

Choice Matrices

CM-01: Voltage Regulator Selection

| Criterion | Weight (%) | R-6212P Buck | TPS62203 Buck | AMS1117 LDO |
| --- | --- | --- | --- | --- |
| Efficiency at 100 mA | 30 | 9 (2.7) | 8 (2.4) | 3 (0.9) |
| Input voltage range | 20 | 9 (1.8) | 8 (1.6) | 7 (1.4) |
| Output ripple | 20 | 7 (1.4) | 8 (1.6) | 4 (0.8) |
| Package availability / footprint ease | 15 | 8 (1.2) | 6 (0.9) | 9 (1.4) |
| Unit cost (qty 5) | 15 | 7 (1.1) | 6 (0.9) | 9 (1.4) |
| WEIGHTED TOTAL | 100 | 8.2 | 7.4 | 5.8 |

CM-02: Temperature/Humidity Sensor Selection

| Criterion | Weight (%) | SHT31 | BME680 | DHT22 |
| --- | --- | --- | --- | --- |
| Temperature accuracy | 30 | 9 (2.7) | 7 (2.1) | 5 (1.5) |
| RH accuracy | 20 | 9 (1.8) | 7 (1.4) | 5 (1.0) |
| I2C interface simplicity | 20 | 9 (1.8) | 8 (1.6) | 3 (0.6) |
| Additional sensing capability | 15 | 3 (0.5) | 9 (1.4) | 2 (0.3) |
| Power consumption | 15 | 8 (1.2) | 6 (0.9) | 7 (1.1) |
| WEIGHTED TOTAL | 100 | 8.0 | 7.3 | 4.5 |

CM-03: I2C Routing Strategy

| Criterion | Weight (%) | Top-layer + GND ref + damping option | Inner-layer no damping | Shielded flex off-board |
| --- | --- | --- | --- | --- |
| Signal integrity (return path continuity) | 35 | 9 (3.1) | 7 (2.5) | 6 (2.1) |
| Noise separation from buck | 25 | 9 (2.2) | 6 (1.5) | 8 (2.0) |
| Ease of debug / rework | 20 | 9 (1.8) | 5 (1.0) | 4 (0.8) |
| Routing complexity | 20 | 7 (1.4) | 8 (1.6) | 3 (0.6) |
| WEIGHTED TOTAL | 100 | 8.6 | 6.6 | 5.5 |

CM-04: PCB Layer Count Selection

| Criterion | Weight (%) | 2-layer ($2.00) | 4-layer ($7.00) | 6-layer (~$25+) |
| --- | --- | --- | --- | --- |
| Signal integrity (dedicated GND plane) | 30 | 4 (1.2) | 9 (2.7) | 9 (2.7) |
| Thermal management (internal planes) | 25 | 3 (0.8) | 8 (2.0) | 9 (2.2) |
| Routing ease / congestion | 20 | 5 (1.0) | 8 (1.6) | 9 (1.8) |
| Manufacturing cost (qty 5) | 15 | 9 (1.4) | 7 (1.1) | 2 (0.3) |
| EMC performance | 10 | 4 (0.4) | 8 (0.8) | 9 (0.9) |
| WEIGHTED TOTAL | 100 | 4.7 | 8.2 | 8.0 |

Conclusion

The weighted choice matrices confirm that the RECOM R-6212P buck converter, Sensirion SHT31 sensor, top-layer I2C routing over a continuous ground plane, and a 4-layer PCB stack-up represent the best trade-offs for this environmental sensor board. Each decision was evaluated against performance, societal, sustainability, usability, aesthetic, and cost criteria derived from the user needs. The 4-layer configuration provides the strongest balance between signal integrity, thermal management, and manufacturing cost at the student-prototype scale. Verification results from TP-VR-001 and TP-SI-001 confirm that the voltage regulation and signal integrity targets are met.