Design & Development Matrix

4-Layer Environmental Sensor PCB - Arduino Nano ESP32

Student ID: ab4647 | Document ID: QMS-FS-001 | Version: 1.0 | Date: March 2026

| Category | User Need | Target Specification | Alternative Solutions | Chosen Solution | Selection Criteria & Weighting | Verification Method | Verification Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Performance | System must regulate a stable 3.3 V supply from a battery input (6-9 V) with minimal ripple to power ESP32 and I²C sensors reliably. | Output voltage: 3.3 V ±2% Ripple: <50 mV pk-pk Input range: 4.5-9 V Efficiency: >85% at 100 mA | 1) RECOM R-6212P buck converter 2) Texas Instruments TPS62203 buck 3) AMS1117-3.3 LDO linear regulator | RECOM R-6212P buck converter | See Choice Matrix CM-01 | TP-VR-001: Measure VOUT with DMM under load; measure ripple with oscilloscope. | PASS - See TP-VR-001 results document. |
| Performance | Sensor must measure temperature and relative humidity with sufficient accuracy for environmental monitoring. | Temperature accuracy: ±0.3 °C RH accuracy: ±2% Interface: I²C Supply: 3.3 V compatible | 1) Sensirion SHT31 (temp + RH) 2) Bosch BME680 (temp + RH + gas + pressure) 3) DHT22 (temp + RH, single-wire) | Sensirion SHT31 as primary; BME680 as secondary for gas/pressure | See Choice Matrix CM-02 | Visual inspection of I²C address response during bring-up. | Deferred - board not yet fabricated. |
| Performance | PCB must maintain signal integrity on I²C bus to ensure reliable communication between ESP32 and sensors. | SDA/SCL routed over continuous ground plane Separation from switching noise >5 mm Optional series damping resistors available | 1) Route I²C on top layer over GND plane with series resistor option 2) Route I²C on inner layer with no damping 3) Use shielded flex cable off-board | Option 1: Top-layer I²C over GND plane with 0 Ω placeholders for optional damping | See Choice Matrix CM-03 | TP-SI-001: Probe SDA/SCL with oscilloscope; check rise time, overshoot, and noise margin. | PASS - See TP-SI-001 results document. |
| Sustainability | PCB laminate material should minimise environmental harm while remaining manufacturable and cost-effective. | RoHS compliant Halogen-free preferred Recyclable via WEEE channels FR-4 baseline with mitigation plan | 1) Standard FR-4 (TG 130-140) 2) Halogen-free FR-4 equivalent 3) Rogers high-frequency laminate | Standard FR-4 with halogen-free alternative noted for future revision | FR-4 chosen for cost, availability, and JLCPCB compatibility. Halogen-free flagged as preferred if supplier supports it at similar cost. | Material declaration review against RoHS directive. | PASS - FR-4 is RoHS compliant. Halogen-free option identified for future. |
| Usability | Board must include test points and debug access to support development, fault-finding, and reuse. | Test points on: GND, VBAT, 3V3, SDA, SCL, EN, FB, VBAT_RAW Minimum pad size: 1.0 mm Accessible with standard probe | 1) Dedicated test-point pads on top layer 2) Exposed vias only 3) Header pins for probing | Dedicated test-point pads on top layer | Test pads are lowest profile, easiest to probe, and do not add connector cost. Header pins add height and BOM cost. | Visual inspection of layout for pad size and accessibility. | PASS - All test points present and accessible. |
| Aesthetic | Board layout should be clean, logically zoned, and suitable for portfolio presentation. | Functional zones clearly separated (power, digital, sensor) Silkscreen labels on all connectors and test points No unrouted nets or DRC errors | 1) Manual placement with logical zoning 2) Auto-place with manual adjustment 3) Compact random placement | Manual placement with logical zoning | Manual zoning gives best control over thermal separation and signal routing. Auto-place does not respect thermal or SI constraints. | DRC pass in KiCad; visual review of layout. | PASS - Zero DRC errors; layout reviewed. |
| Business | PCB must be affordable for a student prototype run (qty 5) using a standard low-cost manufacturer. | Total board cost (qty 5): <$15 USD Manufacturer: JLCPCB or equivalent Standard lead time: 5-7 days | 1) 2-layer PCB ($2.00 per 5, JLCPCB) 2) 4-layer PCB ($7.00 per 5, JLCPCB) 3) 6-layer PCB (~$25+ per 5) | 4-layer PCB at $7.00 (qty 5) + $23.58 shipping = $30.58 total | See Choice Matrix CM-04 | Quote obtained from JLCPCB online calculator. | PASS - Quote confirmed: $30.58 total. |

Choice Matrices

CM-01: Voltage Regulator Selection

| Criterion | Weight (%) | R-6212P Buck | TPS62203 Buck | AMS1117 LDO |
| --- | --- | --- | --- | --- |
| Efficiency at 100 mA | 30 | 9 (2.7) | 8 (2.4) | 3 (0.9) |
| Input voltage range | 20 | 9 (1.8) | 8 (1.6) | 7 (1.4) |
| Output ripple | 20 | 7 (1.4) | 8 (1.6) | 4 (0.8) |
| Package availability / footprint ease | 15 | 8 (1.2) | 6 (0.9) | 9 (1.4) |
| Unit cost (qty 5) | 15 | 7 (1.1) | 6 (0.9) | 9 (1.4) |
| WEIGHTED TOTAL | 100 | 8.2 | 7.4 | 5.8 |

CM-02: Temperature/Humidity Sensor Selection

| Criterion | Weight (%) | SHT31 | BME680 | DHT22 |
| --- | --- | --- | --- | --- |
| Temperature accuracy | 30 | 9 (2.7) | 7 (2.1) | 5 (1.5) |
| RH accuracy | 20 | 9 (1.8) | 7 (1.4) | 5 (1.0) |
| I²C interface simplicity | 20 | 9 (1.8) | 8 (1.6) | 3 (0.6) |
| Additional sensing capability | 15 | 3 (0.5) | 9 (1.4) | 2 (0.3) |
| Power consumption | 15 | 8 (1.2) | 6 (0.9) | 7 (1.1) |
| WEIGHTED TOTAL | 100 | 8.0 | 7.3 | 4.5 |

CM-03: I²C Routing Strategy

| Criterion | Weight (%) | Top-layer + GND ref + damping option | Inner-layer no damping | Shielded flex off-board |
| --- | --- | --- | --- | --- |
| Signal integrity (return path continuity) | 35 | 9 (3.1) | 7 (2.5) | 6 (2.1) |
| Noise separation from buck | 25 | 9 (2.2) | 6 (1.5) | 8 (2.0) |
| Ease of debug / rework | 20 | 9 (1.8) | 5 (1.0) | 4 (0.8) |
| Routing complexity | 20 | 7 (1.4) | 8 (1.6) | 3 (0.6) |
| WEIGHTED TOTAL | 100 | 8.6 | 6.6 | 5.5 |

CM-04: PCB Layer Count Selection

| Criterion | Weight (%) | 2-layer ($2.00) | 4-layer ($7.00) | 6-layer (~$25+) |
| --- | --- | --- | --- | --- |
| Signal integrity (dedicated GND plane) | 30 | 4 (1.2) | 9 (2.7) | 9 (2.7) |
| Thermal management (internal planes) | 25 | 3 (0.8) | 8 (2.0) | 9 (2.2) |
| Routing ease / congestion | 20 | 5 (1.0) | 8 (1.6) | 9 (1.8) |
| Manufacturing cost (qty 5) | 15 | 9 (1.4) | 7 (1.1) | 2 (0.3) |
| EMC performance | 10 | 4 (0.4) | 8 (0.8) | 9 (0.9) |
| WEIGHTED TOTAL | 100 | 4.7 | 8.2 | 8.0 |

Conclusion (50-100 words)

The weighted choice matrices confirm that the RECOM R-6212P buck converter, Sensirion SHT31 sensor, top-layer I²C routing over a continuous ground plane, and a 4-layer PCB stack-up represent the best trade-offs for this environmental sensor board. Each decision was evaluated against performance, sustainability, usability, aesthetic, and cost criteria derived from the user needs. The 4-layer configuration provides the strongest balance between signal integrity, thermal management, and manufacturing cost at the student-prototype scale. Verification results from TP-VR-001 and TP-SI-001 confirm that the voltage regulation and signal integrity targets are met.