Test Protocol: I²C Signal Integrity Verification

| Document ID | QMS-TP-SI-001 |
| --- | --- |
| Version | 1.0 |
| Date | March 2026 |
| Author | ab4647 |
| Related D&D Entry | Performance - Signal Integrity (CM-03) |
| Related QMS Location | QMS/03_Testing/Protocols/ |
| Status | Approved |

## 1. Purpose

This protocol defines the procedure for verifying I²C signal integrity on the SDA and SCL lines of the 4-layer environmental sensor PCB. The test confirms that the routing strategy (top-layer over continuous GND plane, separated from the buck converter region, with optional series damping) produces clean digital edges suitable for reliable I²C communication at standard-mode (100 kHz) and fast-mode (400 kHz) speeds.

## 2. Equipment Required

Oscilloscope with ≥ 100 MHz bandwidth and two 10× probes

Bench power supply (7.4 V nominal, current-limited to 300 mA)

Arduino Nano ESP32 programmed with I²C scanner sketch

SHT31 breakout board connected to I²C header

Test leads

## 3. Test Setup

Step 1: Power the board via VBAT at 7.4 V.

Step 2: Connect the ESP32 to the board. Load I²C scanner firmware.

Step 3: Attach SHT31 breakout to I²C external header (SDA, SCL, 3V3, GND).

Step 4: Connect oscilloscope Ch1 to TP_SDA1 and Ch2 to TP_SCL1, ground clips to TP_GND1.

Step 5: Set oscilloscope to 1 µs/div timebase, 1 V/div vertical, DC coupling.

## 4. Test Procedure

| Test ID | Test Name | Input Conditions | Measurement | Pass Criterion |
| --- | --- | --- | --- | --- |
| T1 | SDA/SCL rise time at 100 kHz | I²C clock = 100 kHz (standard mode) | Measure 10-90% rise time on SDA and SCL. | Rise time < 1000 ns (I²C standard-mode spec) |
| T2 | SDA/SCL rise time at 400 kHz | I²C clock = 400 kHz (fast mode) | Measure 10-90% rise time on SDA and SCL. | Rise time < 300 ns (I²C fast-mode spec) |
| T3 | Overshoot/undershoot on SCL | I²C clock = 400 kHz | Measure overshoot above VCC and undershoot below GND on SCL transitions. | Overshoot < 0.3 V above 3.3 V; undershoot < 0.3 V below 0 V |
| T4 | Noise on SDA during SCL high (crosstalk check) | I²C clock = 400 kHz, SDA held stable | Measure noise amplitude on SDA during SCL transitions. | Coupled noise < 0.4 V (below VIL threshold of 0.3 × VCC ≈ 1.0 V) |
| T5 | Bus operation confirmation | I²C scanner running on ESP32 | Confirm SHT31 detected at expected I²C address (0x44). | Device responds with ACK at address 0x44. |

## 5. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | ab4647 | Initial release. |