Test Protocol: Voltage Regulator Output Verification

| Document ID | QMS-TP-VR-001 |
| --- | --- |
| Version | 1.0 |
| Date | March 2026 |
| Author | ab4647 |
| Related D&D Entry | Performance - Voltage Regulation (CM-01) |
| Related QMS Location | QMS/03_Testing/Protocols/ |
| Status | Approved |

## 1. Purpose

This protocol defines the procedure for verifying that the RECOM R-6212P buck converter on the 4-layer environmental sensor PCB produces a stable 3.3 V output within specification under representative load conditions. This test directly verifies the target specification defined in the D&D matrix for the voltage regulation user need.

## 2. Equipment Required

Digital multimeter (DMM) - resolution ≥ 0.01 V

Oscilloscope with ≥ 100 MHz bandwidth and 10× probe

Bench power supply (adjustable, 4.5-9 V, current-limited)

Resistive load: 33 Ω 0.5 W (≈ 100 mA at 3.3 V)

Test leads and probes

## 3. Test Setup

Step 1: Connect the bench supply to the VBAT input header. Set current limit to 300 mA.

Step 2: Attach the 33 Ω resistive load between the 3V3 test point (TP_3V3) and GND test point (TP_GND1).

Step 3: Connect the DMM across the 3V3 and GND test points.

Step 4: Connect the oscilloscope probe (AC-coupled, 10× attenuation) to TP_3V3 with the ground clip on TP_GND1.

## 4. Test Procedure

| Test ID | Test Name | Input Conditions | Measurement | Pass Criterion |
| --- | --- | --- | --- | --- |
| T1 | DC output voltage at nominal input (7.4 V) | VIN = 7.4 V, load = 33 Ω | Record VOUT on DMM. | VOUT = 3.3 V ± 2% (3.234 V to 3.366 V) |
| T2 | DC output voltage at minimum input (4.5 V) | VIN = 4.5 V, load = 33 Ω | Record VOUT on DMM. | VOUT = 3.3 V ± 2% (3.234 V to 3.366 V) |
| T3 | DC output voltage at maximum input (9.0 V) | VIN = 9.0 V, load = 33 Ω | Record VOUT on DMM. | VOUT = 3.3 V ± 2% (3.234 V to 3.366 V) |
| T4 | Output ripple at nominal input | VIN = 7.4 V, load = 33 Ω, oscilloscope AC-coupled | Record peak-to-peak ripple voltage on oscilloscope. | Ripple < 50 mV pk-pk |
| T5 | No-load output voltage | VIN = 7.4 V, no load (open circuit) | Record VOUT on DMM. | VOUT = 3.3 V ± 5% (3.135 V to 3.465 V) |

## 5. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | ab4647 | Initial release. |