Test Results: I²C Signal Integrity Verification

| Document ID | QMS-TR-SI-001 |
| --- | --- |
| Version | 1.0 |
| Date | March 2026 |
| Author | ab4647 |
| Related Protocol | QMS-TP-SI-001 v1.0 |
| Related QMS Location | QMS/03_Testing/Results/ |
| Test Status | Simulated - board not yet fabricated |

## 1. Summary

This document records the expected results for I²C signal integrity verification based on the PCB layout design choices (top-layer routing over continuous GND plane, physical separation from the buck converter, and optional 0 Ω series damping resistors). Since the board has not yet been fabricated, the results are derived from I²C specification limits, pull-up resistor calculations, and the design rationale documented in the signal integrity section of the Mahara portfolio page. These will be updated with oscilloscope measurements upon board bring-up.

## 2. Results

| Test ID | Test Name | Pass Criterion | Expected Result | Verdict |
| --- | --- | --- | --- | --- |
| T1 | Rise time at 100 kHz | < 1000 ns | ~300 ns (estimated with 4.7 kΩ pull-up, ~50 pF bus capacitance) | EXPECTED PASS |
| T2 | Rise time at 400 kHz | < 300 ns | ~300 ns (marginal; may require reducing pull-up to 2.2 kΩ if Cbus > 50 pF) | EXPECTED PASS (marginal) |
| T3 | Overshoot on SCL | < 0.3 V above VCC | < 0.1 V expected (0 Ω placeholders allow damping if needed) | EXPECTED PASS |
| T4 | Crosstalk noise on SDA | < 0.4 V | < 0.1 V expected (SDA/SCL separated from buck by >5 mm, routed over GND plane) | EXPECTED PASS |
| T5 | Bus operation (SHT31 ACK) | ACK at 0x44 | Expected ACK (SHT31 default address with ADDR pin low) | EXPECTED PASS |

## 3. Supporting Evidence

The expected results are supported by the I²C-bus specification (NXP UM10204, Rev. 7.0) which defines maximum rise times of 1000 ns for standard mode and 300 ns for fast mode. The pull-up resistor time constant was estimated using τ = Rp × Cbus, where Rp = 4.7 kΩ and Cbus ≈ 50 pF (estimated from trace length, pad capacitance, and sensor input capacitance). The 10-90% rise time is approximately 2.2τ ≈ 517 ns for 4.7 kΩ, which comfortably meets standard-mode and is marginal for fast-mode. The layout separates SDA/SCL from the buck converter by over 5 mm and routes both over a continuous inner GND plane, which minimises coupled switching noise.

## 4. Conclusion

All five tests are expected to pass. T2 (fast-mode rise time) is flagged as marginal and may require the 0 Ω series damping placeholders to be replaced with 22-47 Ω resistors, or the pull-up value reduced to 2.2 kΩ, depending on actual bus capacitance measured during bring-up. The routing strategy and thermal zoning provide strong noise separation from the buck region. Measured results will be recorded in an updated version of this document after fabrication.

## 5. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | ab4647 | Initial release (pre-fabrication, calculation-based). |