Test Results: Voltage Regulator Output Verification

| Document ID | QMS-TR-VR-001 |
| --- | --- |
| Version | 1.0 |
| Date | March 2026 |
| Author | ab4647 |
| Related Protocol | QMS-TP-VR-001 v1.0 |
| Related QMS Location | QMS/03_Testing/Results/ |
| Test Status | Simulated - board not yet fabricated |

## 1. Summary

This document records the expected results for voltage regulator verification based on the RECOM R-6212P datasheet specifications and the ANSYS thermal simulation performed on the PCB layout. Since the board has not yet been fabricated, the results below are derived from datasheet typical values and simulation outputs. These will be updated with measured values upon board bring-up.

## 2. Results

| Test ID | Test Name | Pass Criterion | Result | Verdict |
| --- | --- | --- | --- | --- |
| T1 | DC output at 7.4 V input | 3.3 V ± 2% | 3.30 V (datasheet typical) | EXPECTED PASS |
| T2 | DC output at 4.5 V input | 3.3 V ± 2% | 3.28 V (datasheet min-input typical) | EXPECTED PASS |
| T3 | DC output at 9.0 V input | 3.3 V ± 2% | 3.31 V (datasheet typical) | EXPECTED PASS |
| T4 | Output ripple at 7.4 V | < 50 mV pk-pk | ~30 mV pk-pk (datasheet typical with recommended layout) | EXPECTED PASS |
| T5 | No-load output | 3.3 V ± 5% | 3.32 V (datasheet typical) | EXPECTED PASS |

## 3. Supporting Evidence

The expected results are supported by the RECOM R-6212P datasheet (rev. unknown, pp. 1-2, accessed 28 Jan 2026) which specifies typical output accuracy of ±1.5% and output ripple of 30 mV under recommended layout conditions. The ANSYS Icepak thermal simulation confirmed that the buck converter region reaches a peak temperature of approximately 37.8 °C under steady-state conditions with the plane-enhanced layout, which is well within the operating range of the regulator (−40 °C to +85 °C). This supports the expectation that thermal derating will not affect output voltage stability.

## 4. Conclusion

All five tests are expected to pass based on datasheet specifications and simulation results. The R-6212P, when used with the designed layout (adequate input/output decoupling, thermal vias, copper spreading), is predicted to meet all voltage regulation target specifications defined in the D&D matrix. Measured verification will be performed upon board fabrication and will be recorded in an updated version of this document.

## 5. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | ab4647 | Initial release (pre-fabrication, datasheet + simulation based). |